ENME 595: Fluid Mechanics

Laboratory #2

The Effects of Geometry and Mach Number on

Oblique Shock Wave Structure

1.	Objectives

a)	To observe the oblique shocks that form in front of i) a sphere, ii) a pitot tube / turbo-jet entry, iv) a 46° wedge, and vi) a 46° cone.

b)	To measure the oblique shock wave angles in the supersonic flow over the wedge and cone at several Mach numbers.

c)	To show that, for a wedge geometry, oblique shocks remain attached to the wedge only above a certain critical Mach number (or below a critical wedge angle).

e)	To show, for a both the wedge and cone models, whether the flow behind the attached oblique shock waves is sub or supersonic.

f)	To measure that the static pressure distribution over a flat surface that is inclined to the supersonic flow, and to compare the observed pressure distribution with theoretical predictions.

2.	Apparatus

A supersonic jet of air is delivered into the atmosphere through a choked nozzle which has a rectangular exit plane (cross-section 6.8 x 11.0 mm).  The Mach number can be varied by changing the area of the nozzle’s throat using an adjustment knob.  As a result of choking, no variation in the compressed air plenum pressure, P0, can change the mass flow rate through the nozzle for a fixed throat position.  However, decreasing the throat area increases the exit (design) Mach number and the mass flow rate through the nozzle.  Small models with various geometries can be placed into the supersonic jet using the mounting stand.  A shadowgraph displays a picture of the flowfield on a small screen.  This works by focusing a bright light through a series of lenses and then through the test section, causing an image of the flowfield to appear on the screen.  The shadowgraph takes advantage of the fact that the index of refraction of light is directly related to the gas density by n=1+kρ where k is an empirical constant and n is the index of refraction of air.  This results in the gradient of the density being displayed on the screen, so that flow structures such as shock waves appear as dark (somewhat black) lines on their upstream side and white on their downstream side (see Ref. 1 & 2 for more information).

2.	Background and Theory

When a normal shock wave forms in a supersonic flow it causes large increases of entropy and dissipates flow (kinetic) energy into molecular motion.  Normal shock waves can be produced by objects which are placed into a supersonic stream, such as a rounded body like a sphere or even a small pitot tube.  As a result of the dissipative effects that normal shock waves have, subsonic airfoils with rounded noses are inefficient for supersonic aircraft. In addition, the relations that give the Mach number from the knowledge of the ratio P0/P (Pitot pressure / Static pressure) must be corrected because a shock forms in front of the pitot tube.

2.1	Oblique Shock Waves Created by Wedges and Cones

When a wedge (or cone) is placed into a supersonic stream, the oblique shock wave that forms in front of the wedge is either attached (fig. 1a) or detached (fig. 1b) from the wedge, depending on the geometry of the wedge.  When the angle that the wedge deflects the flow is too high, the shock wave will detach.  For the same angle of deflection, oblique shocks produced by a cone are weaker than those produced by a wedge.  This results in a cone being able to deflect the flow through a much larger angle before the shock becomes detached.  Physically, this happens because the cone allows the air to flow around its sides (a wedge does not), and presents a smaller flow disturbance than a wedge.

A wedge in the supersonic flow will produce a family of left and right oblique shock waves (see fig. 1a). If the wedge angle  is smaller than a “critical angle” max the waves will remain attached to the sharp nose of the wedge, for a larger angle the waves will detach (Fig. 1b).  If the waves are attached they form an angle  with the horizontal, called the wave angle.  If the wedge angle tends to zero, the waves will be at an angle  (Mach angle) to the direction of flow (Fig 1c).  The wave angle, Mach angle and the critical wedge angle are a function of the Mach number of the stream.

For both wedges and cones, the shock wave angles can be predicted theoretically. The charts in ref. 1 summarize the theoretical predictions for both geometries.  For wedge geometries (only), one can use the hodographic diagram to predict the shock wave angle.  The hodographic diagram involves a geometric construction, where one basically draws three lines that determine the wave angle.  The procedure is simple:

1.	Draw a line back from the origin at the wedge angle (for this lab 46°/2 = 23°).

2.	Mark the second point where the line drawn in step 1 intersects the curve for the flow Mach number M1. (If it is tangent to the curve or roughly tangent, mark the point of tangency).

3.	Draw a line from the rear intersection of the flow Mach number (M1) curve with the M*-axis through the intersection that you marked in step 2.

4.	Draw a line from the origin that is perpendicular to the line drawn in step 3.

5.	The shock wave angle is the angle that the line drawn in step 4 makes with the M*-axis.

2.2	Linearized flow relations for a flat surface inclined to a supersonic stream

It is fairly easy to theoretically predict the pressure distribution over simple two-dimensional bodies in supersonic streams.  For instance, the static pressure on the surface of a sharp nosed wedge (or supersonic airfoil), for small wedge incidence angles, varies approximately linearly with the angle of incidence.  The proportionality constant (or slope) is a function of only the Mach number.  So the relationship between the pressure drop and the angle of incidence can be written:

DP = f(M)Dq, or for a given Mach number, DP»Dq

It can be shown that if Pat is the atmospheric pressure (or static pressure of the undisturbed stream), and M1 is Mach number of the oncoming supersonic stream, the pressure P on the inclined surface which has an angle  (in radians) to the flow can be expressed as:

P−Pat12ρatV12 = 2M12−1Dq

Physically, the expression relates the coefficient of pressure on the inclined surface to the angle of inclination.  This expression is obtained from a linearization of the flow equations and then finding an infinite series approximation to the solution.  This particular expression has been truncated after the first term.  The above expression is a good approximation for small angles of incidence, and if we eliminate at and V1  it becomes easier to use for calculations.  Using the ideal gas law and the definition of the speed of sound we have:

ρat=gPatgRTat, and gRTat = aat2, V12aat2 = M12

so the denominator is 12ρatV12 = 12gPatM12

one obtains that:

P−Pat12gPatM12 = C1Dq, whereC1 = 2M12 - 1.

The coefficients C1 are tabulated in Table B-8 of Ref. 2.  A slightly better approximation is obtained by truncating the series after the second term, giving a second order approximation:

P−Pat12gPatM12 = C1Dq+C2Dq2.

Coefficients C2 are also tabulated in Table B-8 (Ref. 2).  For large angles of incidence, the pressure coefficient can be obtained more accurately from the oblique shock polar in Pressure-Wedge-Angle coordinates (see Ref. 2).

3.	Procedure

1.	Record the barometric pressure and temperature using the barometer down the hall from the lab.  Make corrections for temperature and latitude.

2.	Mount the sphere, pitot tube, and turbo-jet entry in the test stand and observe the shock wave structure that forms on each model (draw diagrams if necessary).  Perform all the tests at Mach 2.0.

3.	Mount the 46° wedge.  Observe the shock wave that forms on the  model at Mach numbers of 1.5, 2.0, and 2.25.  If the shock wave is attached, record the shock wave angle using the scale on the shadowgraph screen.  If not, record it as “detached”.  Repeat part 3 for the 46° cone.

4.	Mount the second wedge model which has a static pressure measurement hole.  Set the nozzle throat position for Mach 2.0.  (During this test someone must observe the plenum pressure and keep it as steady as possible while measurements are being taken).

4a.	Incline this wedge, using the inclination knob on the test stand, so that the face with the static pressure hole is roughly parallel to the supersonic stream.

4b.	Measure and record the static pressure at this point.

(This value should be close to atmospheric pressure†).

4c.	Using the adjustment knob, rotate the wedge in 2°-2.5° increments. Measure and record the static pressure at each increment.

4d.	Stop taking measurements when the static pressure decreases below the last measurement.

† When the difference between the static hole pressure and the atmospheric pressure (DP) is zero, the face of the wedge is aligned at zero aerodynamical incidence to the stream (αaer=0).  If the pressure measurement at zero geometric incidence (αgeom=0, read from the screen) is not quite the same as atmospheric, then a correction to all of the geometric incidence angles should be applied.  The correction should ensure that DP=0 when αaer=0.

4.	Calculations and Discussion  (Fig. 1, 2, and Fig. 3)

For the wedge and cone models, for each Mach number where the shock was attached, you should determine the angles that the shock waves should make with the wedge and cone models from the theoretical predictions using the hodograph (for wedges only) and tables (for wedges and cones).  For the both the wedge and cone models, you should determine the state variables before and after the oblique shocks.  That is, determine the pressure, temperature, density, sound speed, and Mach number before and after the shock wave (use the angle of the wave in the experiment to get the numbers, rather than the theoretical angles).  Draw conclusions, and discuss differences.  Discuss the physical differences between wedges and cones and explain, from a physical standpoint, why a critical angle or critical Mach number exists.

For the static pressure measurements, the table handed out during the lab is a simple way summarize your observations and calculations.  The table is just like a spreadsheet, where the calculations from one column to the next are performed in succession, depending upon other columns.  (In the lab write-up it is only necessary to show sample calculations for 1 set of the data).  The following table describes each column:

| Column Number | Title (units) | Description |
| --- | --- | --- |
| 1 | Protractor reading (degrees) | The reading from the scale on the shadowgraph screen. |
| 2 | Angular increment (degrees) | The absolute protractor reading, corrected so that the first reading is zero. |
| 3 | Pressure differential (” H20) | Pressure reading from the transducer in the lab |
| 4 | Pressure differential (kPa) | Pressure reading converted to kPa |
| 5 | Aerodynamic angle of incidence (degrees) | Angle of incidence corrected as mentioned in the procedure |
| 6 | Aerodynamic angle of incidence (radians) | The angle must be converted to radians in order to use the formulas in the theory section |
| 7 | None | Square of column number 6 |
| 8 | Pressure coefficient | First order approximation |
| 9 | None | Term to be added to get the second order approximation (see theory section) |
| 10 | Pressure coefficient | Second order approximation |
| 11 | Pressure coefficient | Experimental pressure coefficient |
| 12 | None | Cube of the column number 6 |
| 13 | None | Term to be added to get a third order approximation (see theory section) |
| 14 | Pressure coefficient | Third-order approximation |

Once you have completed the calculations you should compare the predictions of the theory with what you obtained in the laboratory measurements.  Draw conclusions and explain any differences.  Explain, from a physical standpoint, any behaviour that you observe in the results.

References:

(1)	John, J.A., Gas Dynamics, 2nd ed. (1984)

(2)	Shapiro, A.H., The Dynamics and Thermodynamics of Compressible Fluid Flow, Vol. 1, (1953).

· Shawdowgraphs

(1) Chapter 17: “Measurements in Compressible Flow”, Section 17.5		pp. 359-369

· Oblique shock waves on wedges and cones:

(1) Chapter 6: “Oblique Shock Waves”, Sections 6.1, 6.2, & 6.4			pp. 122-135

(2) Chapter 16: Sections 16.1, 16.2							pp. 529-539

· Shock Polar/Hodograph

(2) Chapter 16: Section 16.4							pp. 542-544

· Detached shock waves

(2) Chapter 16: Section 16.5			 				pp. 544-551

· Linearized equations for supersonic flow

(1) Chapter 14: “Linearized Flow”, Sections 14.1-14.4				pp. 279-296

(2) Chapter 16: Section 16.9			 				pp. 561-563

(2) Chapter 14: Sections 14.1-14.3							pp. 427-434

(2) Chapter 15: Sections 15.1, 15.2		 					pp. 462-477

Charts and Tables

(1) Appendix C: “Oblique and Conical Shock Charts”				pp. 388-397

(2) Appendix B: “Table B-8”							pp. 635

Date:		Temperature(°C):		Barometric Pressure (Pat, kPa): 			Mach Number (M):

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protractor Reading (degrees) | Geometric Angle, geom (degrees) | Pressure Differential (“ H2O) | Pressure Differential (kPa) | Aerodynamic Angle, aer (degrees) | Aerodynamic Angle, aer (radians) | (aer)2 | C1(aer) | C2(aer)2 | C1(aer)+ C2(aer)2 | Observed Pressure Coefficient DP12gPatM2 | (aer)3 | C3(aer)3 | C1(aer)+ C2(aer)2+ C3(aer)3 |

For Mach 2.0:	C1 = 1.1550

C2 = 1.4670

C3 = 0.9489

 (air) = 1.40

Conversions:	101.325 kPa = 762.48 mm Hg

762.48 mm Hg = 30.019” Hg = 407.513” H2O

1” H2O = 0.249 kPa