Dan Williams and Luc Bauwens

Department of Mechanical Engineering

The University of Calgary

2500 University Drive N.W.

Calgary Alberta

T2N 1N4

# Abstract

# Introduction

Optimization of computational fluid dynamic computer codes is critical in order to minimize execution time.  Given the high cost of performing large scale simulations on modern supercomputers, one has to carry out optimization in order to save the largest amount of money possible.  In addition, a properly optimized code will minimize the time spent performing simulations of real world problems, hence obtaining the required results much more quickly and efficiently.

We have ported and optimized a code that uses the Flux-Corrected Transport (FCT) algorithm, described in Chapter 4, on two high performance architectures.  The code consists of a set of Fortran subroutines that are collectively referred to by the name of the main subroutine, LCPFCT.  In addition to the main subroutine, several auxiliary subroutines can be used to define the geometry, source terms, and boundary conditions.  LCPFCT can handle cartesian, cylindrical, spherical, or user-defined coordinate systems.  Alternate sets of boundary conditions can be selected by making the appropriate choice of arguments to the subroutine calls.  Besides inflow, outflow, or reflecting wall conditions in any coordinate system, LCPFCT can also handle periodic boundary conditions.  Multidimensional problems are solved using the method of fractional-steps.  The computational grid can be non-uniform, and can move during a time step, allowing the user to perform Lagrangian or sliding rezone calculations.

The optimization of LCPFCT on each computer is the subject of this chapter.  The computers that I have performed the optimization on are very different.  The first is the massively parallel 8,192 processor MasPar MP-1, and the second was the Fujitsu VPX240, a more traditional pipelined vector architecture.  First, we discuss the test problem which we used to benchmark LCPFCT on both architectures.  The problem is a two-dimensional blastwave with periodic boundary conditions.  Second, we discuss the setup, architecture, and optimization process on each computer.  In addition, the results of the optimization process on each computer are shown.

# Two-Dimensional Blastwave Test Problem

The problem used to benchmark LCPFCT on both computers was a two-dimensional blastwave computation with periodic boundary conditions.  This computation involves both supersonic and subsonic flows with interacting shocks and a high degree of symmetry.  The blastwave problem may not be a significant real world problem, but it is a good benchmark for computational fluid dynamic codes.  It is a good benchmark because the solution is very symmetric, it maintains this symmetry for a long time, and a good CFD code should resolve the shockwaves within a few cells while maintaining the symmetry of the solution.  Also, this problem was previously used by Oran et al. (1987), and using the same problem allowed me to compare my results.  In order to construct the two-dimensional solution I used direction splitting, in which the two-dimensional grid is updated one row at a time and then one column at a time.

The computation is initialized with a high-density, high-pressure square of fluid that is 32 cells on each side.  The square is situated in a doubly periodic mesh 128 cells on each side.  The square region in the center of the domain begins with a density fifteen times the background density and a pressure thirty times the background pressure.  The contact surface that defines the interface between the initial high pressure material and the low pressure material is tracked by using an additional species variable in the computations.  This computation is a good benchmark for a CFD model because it should retain its symmetry for a long time, and because periodic or solid wall boundary conditions should give the same solution as long as the symmetry is maintained.  During the simulation, the up-down symmetry is eventually broken by round-off error arising from the limited precision of the floating point calculations.  Symmetry across the 45-degree diagonals is eventually broken by the truncation errors in the direction splitting.  These errors appear to be larger than the roundoff errors.

When the unconfined high-pressure gas is released, a shock forms that races out from the edges of the initial square.  The contact surface, shown in Figure 5.2, closely follows behind the shock.  Figure 5.1 shows the development of the pressure contours for six time steps.  By step 500 the initially square shock has progressed through a circular phase and continues to change shape.  By step 1,500 the shock has reflected from the ends of the computational domain and has begun to recompress the  material inside the contact surface.  As time proceeds on to step 10,000 the shocks become progressively weaker, and become oriented parallel with the sides of the domain.

Whereas the shock patterns become simpler as time progresses, the vorticity caused by the shock interactions with the contact surface warps the interface into increasingly complex patterns.  The vorticity is generated by the baroclinic source term in the vorticity equation.  This term is nonzero when the gradients of density and pressure are not aligned. The misalignment is created by the shock reflecting from the corners of the domain, which weakens the shock.  The shocks reflect back through the interface creating more vorticity.

# The MasPar MP-1

The MasPar system architecture, shown in Figure 5.3, includes a processor element array, an array control unit, and a UNIX workstation as a front end.  The front-end manages program execution and user interface.  When there is a need for parallel execution, the front-end sends the program for execution to the processor array.  The 8,192 processors are organized on a two-dimensional rectangular array topology (128×64).  The MasPar implements what is called a single-instruction, multiple-data (SIMD) architecture.  In a SIMD architecture, all the processors simultaneously perform the same operation on different data as one single stream of instructions is broadcast to all processors by the array control unit.  The MasPar MP-1's theoretical peak performance is 650 Mflop/s single precision and 290 Mflop/s double precision.

#### µ §

Figure 5.3: A simple diagram of the MasPar's layout

µ §

#### Figure 5.4: Setup of the The X-Net commuication lines

A key issue to keep in mind when programming any SIMD massively parallel architecture is minimization of communication between processors.  If communication is not taken into consideration while programming the MasPar, the performance penalties can be quite drastic.  Communication on the MasPar can be divided into three different categories.  First, there is communication between the array control unit and the processor array, where the array control unit broadcasts data or instructions to all processors in the array simultaneously.  Second, there is interprocessor communication which is required when a processor requires data that is not resident in its local memory.  Adjacent processors communicate through the X-Net.  The X-Net, shown in Fig. 5.4, is an eight way, two-dimensional toroidal mesh.  It is set up so that the center processor "C" can communicate along the eight directions around it, shown by dotted lines.  In this case, the performance penalty is low because X-net communication is fast.  Communication between arbitrary processors is done with a hierarchical crossbar called the global router.  The router will be used if the center processor "C" in Fig. 5.4 must communicate with one of the shaded processors.  While router communication is efficient,  it is much slower than X-net communication -- the X-net has approximately 16 times the bandwidth of the router.

The final, and most costly, type of communication is array sloshing between the front-end and the processor array.  Array sloshing has a profound effect on performance, and occurs in several circumstances.  First, if an array that has been allocated on the processor array is accessed in a serial (FORTRAN 77) manner, it will be sloshed to the front-end.  To avoid this situation, serial access on the processor array should be avoided.  Likewise, when an array has been allocated on the front-end, and a subroutine is called that uses this array in a parallel context, the array will be sloshed from the front-end to the processor array and back again when the routine exits.  To avoid this, compiler directives can be used to force allocation on the processor array at declaration time.  Even more costly than the latter two cases is the sloshing of a COMMON block of arrays.  This can be avoided in the same manner as for individual arrays.

## Optimization on the MasPar

The MasPar can be programmed in either MPL, a parallel extension to C, or MPF, an implementation of a subset of Fortran 90.  I used version 3.012 of MasPar's high performance Fortran compiler "mpfortran" or MPF.  In MPF, parallel operations are expressed with the Fortran 90 array extensions.  Arrays are treated as unitary objects rather than requiring them to be iterated through one element at a time, as in standard Fortran 77.  MPF generates code for both the front end and the processor array, effectively making the details of the architecture transparent to the programmer.  However, programming style will directly affect performance.

Optimization of FCT on the MasPar required two main issues to be addressed: making effective use of the processor array, and minimization of communication.  In SIMD architectures, operations performed on a subset of the processor array, such as single lines or columns, or boundary nodes, cost as much in cycles as operations on the whole array.  Thus, constructs dealing, for instance, with the boundaries separately from the main arrays can easily double the computation time, and they are advantageously concatenated with the main operations.  In addition, unwanted interprocessor communication can occur if arrays are not correctly allocated on the processor array.  It was critical to ensure that all arrays were properly aligned on the processor array in order to minimize communication overhead.

Porting the code involved getting the code to run on the processor array, and optimizing the code on the processor array.  First, since the subroutines were originally  written in FORTRAN 77, they were converted to Fortran 90 using the MasPar version of Pacific-Sierra's VAST-2.4.01L (DPVAST) translator.  The translator searches the FORTRAN 77 code for scalar "do" loops and converts them to the Fortran 90 array notation, which is understood by the processor array.  The code as translated by DPVAST ran very poorly -- the temporal performance was much less than one time step per second.  Optimization of the code was carried out in several stages described below, and the results of optimization are summarized in Table 5.1.

Stage one dealt with COMMON block allocation.  The original blocks contained both scalar and array data, and as a result, the compiler allocated them on the front-end.  Since the arrays in the blocks were used in Fortran 90 array constructs,  the whole block had to be sloshed to the processor array.  By removing all COMMON blocks, and passing the block elements as parameters to the subroutines, we improved the temporal performance by a factor of two.  This was still unsatisfactory.

Stage two consisted of altering the implementation of direction splitting.  The original LCPFCT subroutines were one-dimensional.  Two-dimensional problems were handled by calling the one-dimensional subroutines row by row, and then column by column.  This minimizes scratch memory -- a desirable feature on a vector machine, but leads to poor performance on a SIMD architecture.  For example, for a 128×128 grid, a row or column array will contain only 128 elements.  When the one-dimensional subroutines are called with a 128 element array only 128 processors will be used at a time, resulting in a performance that is less than 1.0% of the peak performance of the machine.  The code was rewritten such that all the rows were dealt with in a single operation and then all the columns.  This can be done because adjacent rows or columns are independent of one another during this computation.  This stage improved the temporal performance to approximately 3 tstep/s.

The profiler was used in stage three to find out which parts of the code used the most CPU time.  The profiles showed that approximately 70% of the time was spent in the LCPFCT subroutine, and a large percentage of the time was spent in calculating the boundary conditions.  The DPVAST version of the code used two one-dimensional arrays to store the results of the boundary calculations. These arrays could not be properly aligned with the main two-dimensional arrays.  To eliminate the communication penalty that results from this situation, the code that dealt with the boundary calculations was rewritten, and the results of the boundary calculations were stored in the main two-dimensional arrays.  As a result of the modifications performed in this stage the temporal performance improved to 4.55 tstep/s.

Stage four consisted of replacing some of the subroutine array parameters with COMMON blocks.  As a result of eliminating the COMMON blocks in stage two of the optimization, a significant amount of overhead had been created from passing a large number of array parameters to the subroutines.  The original COMMON blocks contained both array and scalar data, COMMON blocks that contain only arrays and no scalar data will be properly allocated on the processor array.  Replacing some of the subroutine array arguments with COMMON blocks reduced the overhead.  This stage further improved the temporal performance to 5.56 tstep/s.

In stage five, I eliminated all router operations.  The code profiles indicated that there was router communication overhead, because some of the subroutine calls included arguments that were parts of arrays.  This situation is handled by copying into a temporary array through use of the router, which is time consuming.  The code was modified to eliminate the router operations and temporal performance increased to 5.88 tstep/s.  This was only a modest improvement in performance because the modification was made to a subroutine that only consumed about 3 percent of the total execution time.

Further improvement was obtained in stage six by aligning all array allocations.  The current compiler implementation maps arrays directly on the processor array based upon declaration indices, without any optimization attempt.  Some of the arrays needed to include boundary nodes, while some others did not. The other arrays had been declared with their minimal size, which caused unnecessary shifting of whole arrays to occur.  This is costly and can be avoided by proper array declaration.  By changing the declaration indices, the array allocations were aligned, and the temporal performance improved to about 7 tstep/s.

## Optimization Results and Discussion

Table 5.1 presents a summary of the effects that each stage of optimization had upon the performance of FCT.  The benchmark performance2 was calculated as suggested by Hockney (1993).  The largest improvement in performance was obtained in stage two, which required the code to be completely rewritten.  The other stages resulted in modest improvements, but were much less time consuming than stage two.  Overall, optimization of FCT on the MasPar was by far the most time-consuming task, while the translation process (DPVAST stage) was relatively straightforward.  Table 5.1 effectively demonstrates the significance of optimization.  By modifying the way in which the code was written I have improved the performance by a factor of approximately 240 over the DPVAST version.  At this point however, further improvement would require rewriting a fully two-dimensional version of FCT for the MasPar that does not use direction splitting.

| Table 5.1: Effect of the Optimization (Resolution: 128´128) |  |  |  |
| --- | --- | --- | --- |
| Stage | Temporal Performance (tstep/s) | Benchmark Performance (Mflop/s-64 bit) | Time to perform optimization (hr) |
| DPVAST | 0.03 | 0.3 | 2.0 |
| 1 | 0.06 | 0.7 | 3.0 |
| 2 | 3.03 | 35 | 15.0 |
| 3 | 4.55 | 46 | 3.0 |
| 4 | 5.56 | 63 | 3.0 |
| 5 | 5.88 | 68 | 2.0 |
| 6 | 7.14 | 81 | 1.0 |

| Table 5.2: FCT Benchmark Performance for Different Grid Sizes on the MasPar MP-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Grid Size | Single Precision |  | Double Precision |  |
|  | RB (Mflop/s) | % peak | RB (Mflop/s) | % peak |
| 128×64 | 146 | 24 | 90 | 31 |
| 128×128 | 108 | 18 | 81 | 28 |
| 256×128 | 126 | 21 | 87 | 30 |
| 256×256 | 138 | 23 | 93 | 32 |
| 512×512 | 174 | 29 | Insufficient memory |  |

Table 5.2 shows the benchmark performance achieved with the optimized version of the code.  The first row of Table 2 (128×64) corresponds to a problem with an array size that can be mapped optimally onto the processor array.  In this case there is a minimum in communication overhead because none of the arrays have to be mapped into multiple layers of processor memory.  For the 128×128 case a drop of about 25% in single precision, and about 10% in double precision performance is observed.  This would be due to the  communication overhead created by one extra layer of memory having to be allocated.  Table 5.2 also shows the percentage of the peak performance attained by FCT.  Since I have implemented two-dimensional FCT computations using direction-splitting, the peak performance that the code could attain is reduced by at least a factor of two.  This is because the rows and columns of the grid are dealt with separately.  With a fully two-dimensional version of FCT this problem could be fixed, but would require substantial rewriting of our original code.  Taking  this into consideration the results are quite good.

Figure 5.5 shows the dependence of benchmark performance on the problem size.  After the initial drop in benchmark performance, when changing from an 128´64 to a 128´128 grid, the performance increases at an approximately logarithmic rate as the problem size increases. This is because the ratio of floating point operations to communication overhead is increasing.  The incremental changes in benchmark performance seem to be much more dramatic for the single precision than double precision, because communication overhead is independent of the precision.

µ §

Figure 5.5:  Dependence of benchmark performance on the problem size

Timings and floating point performance for FCT on various architectures are presented in Table 5.3.  The data for the Connection Machine CM-2 and the Cray Y-MP was obtained from Oran et al. (1987).  Timings are shown for two typical vector supercomputers, the CRAY Y-MP and the Fujitsu VPX240.  The timing for the MasPar as compared to the Cray is competitive, with the MasPar approximately 7% faster.  The Fujitsu is a much faster machine than either the Cray or the MasPar, with a theoretical peak performance of 2.5 Gflop/s.

| Table 5.3: Benchmarks for the Two-Dimensional Blast Problem (Resolution: 128×128) |  |  |
| --- | --- | --- |
| Computer Type | Temporal Performance (tstep/s) | Benchmark Performance (Mflop/s - 64 bit) |
| MasPar MP-1 (8192K) | 7.25 | 81 |
| CM-2 (8192K) | 5.00 | 57 |
| CRAY Y-MP (1 CPU) | 6.64 | 76 |
| Fujitsu VPX240 | 52.7 | 586 |
| IBM RS6000/950 | 0.96 | 11 |
| HP 9000/710 | 0.55 | 6.2 |

Another parallel architecture was also included for comparison.  The Connection Machine CM-2 has a SIMD architecture similar to the MasPar.  The MasPar timing is 43% faster than the Connection Machine.  FCT on the Connection Machine was written in a parallel implementation of C called C* (C star) by Oran et al. (1987), while the code was written in Fortran 90 on the MasPar.  The final two platforms presented for comparison are both single processor RISC workstations

Further optimization of the code could be obtained by writing a fully two-dimensional version of LCPFCT, but this would require starting over from scratch.  Due to the limited access time to the MasPar this step was not performed.  The results only considered rectangular domains.  To avoid the drastic performance degradation involved with simulating flow through more complex domains, techniques such as domain decomposition, and mapping of rectangular sub-domains on the processor array topology would be required.  In any event, finite-difference algorithms are arguably not the most suitable for complex geometries.

# The Fujitsu VPX240

In contrast to the MasPar MP-1, which achieved its performance by using a large number of simple processors, the Fujitsu VPX240 is a more traditional supercomputer that uses multiple vector pipelines to get high floating point operation rates.  The VPX240 has four add-multiply pipelines, and can make eight floating point operations per clock cycle.  Hence, the VPX240 is capable of a peak theoretical performance of 2.5 Gflop/s.  The Fujitsu has one similarity to the MasPar in that multiple data in the pipelines have the same operations performed on them regardless of the values of that data.  So in a sense we could call the Fujitsu a SIMD parallel architecture as well, although it is not massively parallel.

µ §Figure 5.6: Block Diagram of the Fujitsu VPX240 architecture

Figure 5.6 shows a schematic layout of the VPX240.  The computer consists of several units, each with its own purpose.  The vector unit is the part of the computer that could be called SIMD.  It is responsible for performing operations on array data.  The main storage unit is simply the main memory of the computer.  The system storage unit is secondary memory that is primarily used for I/O operations.  Finally, the scalar unit performs all non vector operations.	Optimal performance on any vector architecture is obtained for some type of simple kernel.  On one of the first vector computers, the Cray 1, this was adding or multiplying two vectors (where a vector is a one-dimensional array of data).  With the advent of pipelines in the Cray X-MP the optimal kernel became a combined multiply/add operation.  This operation is referred to as a "pipe".  In contrast to the combined multiply/add operation, the optimal kernel on the VPX240 is the matrix multiply computation.  Figure 5.7 shows a small FORTRAN program which performs a 1024´1024 (1024 is the optimum vector length for the Fujitsu) matrix multiply.  Timing calls inserted around the do loop gave a floating point rate of 2.38 Gflop/s.  The VOCL compiler directive tells the compiler to unroll the loop to a depth of four.  Unrolling the inner loop will increase the number of floating point operations in the inner loop to eight, which matches the four pipe architecture of the VPX240.  The maximum of 2.5 Gflop/s cannot be achieved due to overhead associated with the do loop.  In fact, if you use the CPU time used by the program to calculate the floating point rate, you get about 1.83 Gflop/s.  The slowdown is strictly due to overhead.

program fast

integer n

parameter (n=1024)

integer i, j, k

double precision c(n,n),a(n,n),b(n,n)

do 4 j = 1,n

*VOCL LOOP, UNROLL(4)

do 4 k = 1,n

do 4 i=1,n

4			c(i,j) = c(i,j) + a(i,k)*b(k,j)

end

Figure 5.7:  Source code for the optimal kernel on the Fujitsu VPX240

The key to obtaining good performance on the Fujitsu is to make optimal use of the vector pipelines.  Ensuring that 8 floating point operations are being performed per clock cycle, and that the pipelines are as full as possible at all times is critical.  In terms of programming structure this manifests itself in several areas.  First, to get the maximum amount of floating point operations per cycle, "do loops" must contain an adequate amount of work within them.  It is better to minimize the number of loops while maximizing the amount of work within a loop, than to have several loops with only a little bit of work in each.  Generally, when using unstructured grids, vector length is completely dependent on the mesh size.  As the mesh is made finer, the vector lengths increase, and hence the floating point operation rate will go up.

## Optimization of FCT on the Fujitsu VPX240

Optimization of LCPFCT on the Fujitsu also consisted of several stages.  Keeping in mind the architectural issues, I proceeded with the optimization.  The Fujitsu can be programmed in several different languages including C, FORTRAN, and assembly language.  I used standard FORTRAN 77 along with the "frt" compiler.  In order to begin optimization, I had to establish a baseline performance.  The original LCPFCT subroutines were one-dimensional, with multidimensional problems handled through direction splitting.  The baseline temporal performance was about 13 tstep/s on a 128´128 grid (I will quote temporal performance only for this grid size within the text).  The results of the optimization process for several grid sizes is presented in Table 5.4.

Stage one of the optimization process involved concatenating several do loops in order to increase their computational intensity.  The original LCPFCT subroutines were optimized for a CRAY X-MP, and consisted of many do loops operating on single arrays.  This is good for the single pipe architecture of the CRAY X-MP, but does not take full advantage of the multiple pipes on the Fujitsu.  Concatenating the do loops in LCPFCT required one extra array, however the temporal performance increased to 15.18 tstep/s.

In stage two I altered the implementation of the direction splitting method in exactly the same way as on the MasPar.  Since the original subroutines were one-dimensional, calculation of a two-dimensional solution consisted of copying each row of data from the two-dimensional array to a one-dimensional array, which was then transported and copied back to the two-dimensional array.  This process was then repeated for the columns.  This procedure involves copying large amounts of data around in memory -- a process the Fujitsu is not very efficient at.  I rewrote the subroutines to operate directly on the two-dimensional arrays and the temporal performance increased to 24.66 tstep/s.

In stage three I simply ensured that all loops that could be fully vectorized did actually vectorize.  The process of getting all the loops to vectorize was quite straightforward, and only required rearrangement of some of the code.  If a loop is not completely vectorized, the computations within it are performed by the scalar unit which is much slower than the vector unit.  The modifications in this stage increased performance to 35.87 tstep/s.

Stage four involved optimizing memory access.  As a result of stage two, some of my loops were not accessing memory with unit stride.  Using non-unit strides causes memory bank conflicts and hence significant performance degradation.  Accessing two-dimensional arrays in FORTRAN consists of using two nested do loops.  In terms of my source code, fixing this problem consisted of accessing the left index of the two-dimensional arrays with the inner do loop.  Fixing this problem caused the temporal performance to increase to 49.26 tstep/s.

Finally, stage five was originally carried out only to decrease the amount of memory needed, but some performance improvement was also obtained.  After stage two of the optimization, the grid dependent variables had been changed from one-dimensional to two-dimensional.  This was good for a computer such as the MasPar, since it minimized any communication overhead, but was not good for the Fujitsu.  I rewrote the grid dependent subroutines as one-dimensional to save memory, and there was a performance increase because unnecessary operations were not being performed anymore.  This stage further improved the temporal performance to 52.74 tstep/s.

## Results and Discussion

Table 5.4 shows the results of the optimization process for three different grid sizes.  Unlike the MasPar, where one stage obviously gave the greatest performance gain, no particular stage seems to stand out as offering the most improvement.  All of the stages seem to offer more or less equal gains in performance.  However, not each of the stages required the same amount of time to implement.  Stage two required significant restructuring and rewriting of the code, and took the longest.  All the other stages took much less time than stage two, but still offered similar performance gains.  As a result of these optimizations the performance increased by a factor of about 4, 3.5, and 3 respectively, for each grid size.  Further improvement could be obtained by concatenating code at the subroutine level, but only at the cost of increased memory usage.  Minimizing the amount of memory usage becomes more critical when performing three-dimensional simulations.

| Table 5.4: Effects that each stage of optimization had for various grid sizes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 128´128 |  | 256´256 |  | 512´512 |  |
| Stage | RT (tstep/s) | RB (Mflop/s) | RT (tstep/s) | RB (Mflop/s) | RT (tstep/s) | RB (Mflop/s) |
| Baseline | 13.40 | 156 | 4.39 | 204 | 1.32 | 245 |
| 1 | 15.18 | 177 | 4.98 | 231 | 1.47 | 274 |
| 2 | 24.66 | 287 | 5.59 | 260 | 2.31 | 429 |
| 3 | 35.87 | 417 | 6.68 | 310 | 2.81 | 515 |
| 4 | 49.26 | 573 | 13.42 | 624 | 3.68 | 684 |
| 5 | 52.74 | 586 | 15.50 | 688 | 3.97 | 705 |

Table 5.5 shows the benchmark performance achieved with the optimized version of the code.  At the highest benchmark performance value (for a 512´512 grid) the code is getting about 29% of the peak performance of the Fujitsu.  This figure is normal for most codes in general use today.  Again, like on the MasPar, since I have implemented two-dimensional FCT computations using direction-splitting, the peak performance that the code could attain is reduced by at least a factor of two.  Table 5.5 also shows the percentage of time spent in the vector unit.  This figure indicates only how much time was spent in the vector unit as a percentage of the total CPU time.  Even though the percent vectorization is quite high, if the vector unit is not being used efficiently (8 flops per clock cycle) the benchmark performance suffers.  Some more gains could be obtained by merging code at the subroutine level, rather than the loop level, but only at the cost of increased memory usage.

| Table 5.5: Performance of the fully optimized LCPFCT on the Fujitsu VPX240 (500 timesteps) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid Size | CPU Time (s) | VU Time (s) | % Vectorization | RT (tstep/s) | RB (Mflop/s) |
| 128´128 | 9.48 | 9.16 | 96.59 | 52.74 | 586 |
| 256´128 | 17.25 | 16.81 | 97.44 | 28.98 | 644 |
| 256´256 | 32.25 | 31.77 | 98.50 | 15.50 | 688 |
| 512´256 | 63.09 | 62.20 | 98.58 | 7.92 | 703 |
| 512´512 | 125.89 | 124.06 | 98.54 | 3.97 | 705 |

µ §

Figure 5.8:  Dependence of benchmark performance on problem size

Figure 5.8 shows the dependence of benchmark performance on the problem size.  The benchmark performance increases in an inverse exponential manner.  That is, it approaches a peak value, but never quite attains that value.  The gain in benchmark performance is more dramatic when changing from a small mesh size to the larger mesh sizes.  This is because once the mesh size approaches the point where the pipelines are full, further increases in mesh size do not effect the performance as much.  The optimization process has the effect of shifting this curve in the vertical direction.

µ §

Figure 5.9:  Dependence of temporal performance on problem size

Figure 5.9 shows the dependence of temporal performance on the problem size.  The temporal performance decreases in an inverse exponential manner.  That is, it approaches a minimum value, but never quite attains that value.  This occurs for the same reason stated for the behavior of the benchmark performance.  The optimization process has the effect of shifting this curve in the vertical direction also.

Given the results that were obtained on both machines it is obvious that optimization of CFD codes is critical in order to maximize performance.  As a result of a few days work on optimization I can now run my code up to four times faster for the same mesh size, or I can use a mesh size four times finer and get much better results in the same amount of time as the baseline code.  In this manner I have also maximized the spending power, for CPU time, of my money.  This is the main point of optimization.

Hockney (1993) defines temporal performance as the inverse of the execution time [RT = T-1(N;p)], where N is the problem size and p is the number of processors.  Temporal performance is measured in solutions per second (sol/s) or time steps per second (tstep/s).  It is a good metric for comparing different algorithms when solving a certain benchmark problem because it tells you which algorithm solves the problem the fastest.

2 	Hockney (1993) defines benchmark performance as the ratio of the benchmark floating point operation (flop) count Fb(N) and the execution time T(N;p) [ Rb = Fb(N)/T(N;p) ].  The benchmark performance generally has units of million floating point operations per second (Mflop/s).  The benchmark flop counts for both the MasPar and Fujitsu codes were obtained by counting operations as suggested by Hockney [4] (+, -, × = 1 flop; ¸, Ö = 4 flop; exp, sin, cos, etc... = 8 flop; if (x .rel. y) = 1 flop).